| Module Name: | Interrupt Controller |
| --- | --- |
| Inputs: | Clk, n_rst, hw_interrupt_requests, addr, ren, wen, wdata, rambusy |
| Outputs: | Interrupt_service_request, rdata |
| Parameters: | N_interrupts, N_interrupt_registers |

The Interrupt Controller is the top level module for the PLIC. The PLIC takes in hardware interrupt requests and serves them, along with an interrupt ID, to the processor. The Interrupt Controller is currently built for an address width of 32 bits and stores information in 32 bit registers. If the current microcontroller is not 32 bits then external shifting logic will have to be added.

The module’s addr, ren, wen, rdata, wdata, and rambusy ports are used to communicate to the registers. This module uses global address 0x00 as local address 0x00 so external address shifting is required.

Hardware interrupt channels are mapped to the hw_interrupt_requests wires. The index that each interrupt is assigned to is used for interrupt identification.

The parameter N_interrupts determines the number of hardware interrupt channels. The parameter N_interrupt_registers determines the number of interrupt registers used to store interrupt channels. This should be at least roof(N_interrupts/32).

| Module Name: | Interrupt Enable Registers |
| --- | --- |
| Inputs: | Clk, n_rst, enable_mask_below_val, interrupt_priority_regs, addr, ren, wen, wdata, rambusy |
| Outputs: | Interrupt_masks, rdata, wdata, addr_valid |
| Parameters: | N_interrupts, N_interrupt_registers |

The interrupt enable register handles logic controlling which registers are masked. The module handles status registers related to masking individual interrupts as well as interrupt masking when the disable low priority interrupts module is enabled.

Enable_mask_below_val is the data line that determines whether the module will mask interrupts whose priority value is below the value stored in the PLIC_mask_below_value registers. When enable_mask_belo_val is set to 0 then the module will revert to the current PLIC_mask_interrupt register settings.

The interrupt_masks output register contains a binary representation of which interrupts are to be masked. The index 0 corresponds to hardware interrupt 0. A value of ‘1’ for a channel indicates that the interrupt is masked. A value of ‘0’ indicates that the interrupt is not masked.

The module’s addr, ren, wen, rdata, wdata, and rambusy ports are used to communicate to the registers.

The parameter N_interrupts determines the number of hardware interrupt channels. The parameter N_interrupt_registers determines the number of interrupt registers used to store interrupt channels. This should be at least roof(N_interrupts/32).

| Module Name: | Interrupt Pending Priority Registers |
| --- | --- |
| Inputs: | Clk, n_rst, interrupt_requests_masked, active_interrupt, interrupt_serviced, enable_module, addr, ren, wen, wdata, rambusy |
| Outputs: | Pending_interrupts, interrupt_priority_regs, rdata, addr_valid |
| Parameters: | N_interrupts, N_interrupt_registers, Base_address |

The interrupt pending and priority registers module handles registers controlling interrupt priorities as well as the interrupt pending registers. The interrupt priority registers indicate the priority of each hardware interrupt channel and the interrupt pending registers indicate which registers are in the queue to be serviced.

Interrupt_requests_masked is the output of the interrupt request channels after they have been masked. A 1 value for a given channel indicates the register pending bit should be set, and a 0 value indicates that the bit should remain as it is.

The enable_module bit enables storing of pending interrupt requests. Setting this value to 0 will mask all requests and ignore any requests until the bit is set to 1.

Active_interrupt is a register that indicates which interrupt is currently being serviced. This register controls what interrupt is set to inactive when interrupt_serviced is pulsed.

Interrupt_service is a wire which accepts a pulse that will set the currently active interrupt as serviced.

Pending interrupts is the register that contains all interrupt pending bits, mapped 1:1 for each hardware channel. Bit 0 maps to hardware interrupt channel 0

Interrupt_priority_regs is the interrupt priority value of each hardware interrupt. Index 0 maps with hardware interrupt channel 0.

The module’s addr, ren, wen, rdata, wdata, and rambusy ports are used to communicate to the registers.

The parameter N_interrupts determines the number of hardware interrupt channels. The parameter N_interrupt_registers determines the number of interrupt registers used to store interrupt channels. This should be at least roof(N_interrupts/32).

| Module Name: | Interrupt Priority Resolve |
| --- | --- |
| Inputs: | Interrupt_priorities, pending_interrupts |
| Outputs: | Active_interrupt, active_interrupt_info, interrupt_request |
| Parameters: | N_interrupts, SYSBITS |

The interrupt priority resolve register handles sending the highest priority interrupt index to the status registers for the CPU to read.

Interrupt_priorities is a series of registers containing the priority value for each interrupt. Pending_interrupts is the register containing the status of each hardware interrupt. A logic value of 1 indicates that the interrupt is to be serviced.

Active interrupt indicates the currently active interrupt to be serviced.

Active interrupt info is the index of the hardware interrupt, encoded in binary. For example, if channel 5 is serviced, active interrupt would output b1010. Hardware interrupt 0 is mapped to value 1.

Interrupt request is the pulse sent to the processor to indicate that an interrupt requests servicing.

N_interrupts is the number of hardware interrupt bits in the module.

SYSBITS is the memory size of the hardware. Setting this value may require additional translation circuitry.

| Module Name: | Interrupt Request Registers |
| --- | --- |
| Inputs: | Clk, n_rst, interrupt_requests_in |
| Outputs: | interrupt_requests |
| Parameters: | N_interrupts |

Interrupt request registers translates hardware interrupt requests into pulses to be sent to other components in the submodule.

Interrupt_requests_in are the pads connected to hardware interrupt channels.

Interrupt_requests is a map of the output pulses to the module.

N_interrupts is the number of interrupts in the module.

| Module Name: | Rdata Arbiter |
| --- | --- |
| Inputs: | Rdata1, rdata2, rdata3, addr_valid1, addr_valid2, addr_valid3 |
| Outputs: | rdata |
| Parameters: |  |

Rdata arbiter multiplexes the multiple streams of possible read data that can be output and places it in a single rdata bus.

Rdata1 is the rdata register of the first module.

Rdata2 is the rdata register of the second module.

Rdata3 is the rdata register of the third module.

Addr_valid1 tells the arbiter whether to output rdata1.

Addr_valid2 tells the arbiter whether to output rdata2.

Addr_valid2 tells the arbiter whether to output rdata3.

Rdata is the multiplexed output of the three modules. If no address is valid, the arbiter will place rdata in a Hi-Z state.

| Module Name: | Register Mask |
| --- | --- |
| Inputs: | Interrupt_requests, interrupt_masks |
| Outputs: | interrupt_requests_masked |
| Parameters: | N_interrupts |

Register mask prevents masked interrupts from triggering an interrupt request.

Interrupt requests is the register of interrupt request pulses.

Interrupt masks is the register of interrupt masks. A value of 1 means that the interrupt is masked.

N_interrupts is the number of hardware interrupt channels.

| Module Name: | Interrupt Service Status Register |
| --- | --- |
| Inputs: | Clk, n_rst, active_interrupt_info, interrupt_request, active_interrupt, addr, ren, wen, wdata, rambusy |
| Outputs: | Interrupt_serviced, mask_all, enable_mask_below_val, enable_module, interrupt_request_pulse, rdata, addr_valid |
| Parameters: | N_interrupts, N_interrupt_registers, Base_address |

The interrupt service/status register contains logic for controlling the status registers. This module handles global enable/disable module registers as well as interrupt information and the interrupt acknowledge registers.

Active_interrupt_info is the channel of the current active hardware interrupt. Channel 0 is mapped to a value of 1.

Interrupt_request indicates that an interrupt is pending.

Interrupt_serviced is a pulse that indicates the CPU has serviced the active interrupt.

Mask_all disables all interrupts from activating the interrupt register.

Enable_mask_below_val enables the priority-level based masking feature.

Enable_module is the current status of the module. If set to 0, then the module should be disabled until it is set back to 1 in the status register.

Interrupt_request_pulse is the pulse that is sent to the CPU to indicate that an interrupt is pending.

The module’s addr, ren, wen, rdata, wdata, and rambusy ports are used to communicate to the registers.

The parameter N_interrupts determines the number of hardware interrupt channels. The parameter N_interrupt_registers determines the number of interrupt registers used to store interrupt channels. This should be at least roof(N_interrupts/32).

The parameter Base_address indicates what shifted address this module should respond to.